# BulletCam HL (5 Mp/2.8 mm)

Wired AI-powered security IP camera with a 110° viewing angle, hybrid illumination, TrueWDR, microphone, and PoE/12 V. For outdoor and indoor use.

| ### Camera | ### Image sensor 5 MP CMOS, 1/2.8" type ### Lens focal length 2.8 mm ### Maximum aperture f/1.6 ### Lens iris type fixed ### Lens focal type fixed ### Field of view horizontal: 100–110° vertical: 50–60° diagonal: 125–135° ### DORI distance - Detection: 65 m - Observation: 26 m - Recognition: 13 m - Identification: 6.5 m ### Maximum illumination distance - white LED: up to 50 m - IR LED: up to 50 m Intelligent lighting control automatically adjusts the intensity of IR and white LEDs in real time to prevent overexposure. This ensures clear visibility of objects at varying distances — whether close to or far from the camera — in low-light conditions. ### Minimum illumination color: 0.005 lx B/W: 0 lx ### Signal-to-noise ratio (SNR) more than 52 dB |
| --- | --- |
| ### Video | ### Resolution - 5 MP (2880 × 1620) - 4 MP (2560 × 1440) - 3 MP (2304 × 1296) - 2 MP (1920 × 1080) - 0.6 MP (1024 × 576) ### Frame rate up to 25 fps ### Image rotation 0°, 180° ### Video codec H.265, H.264 ### Dynamic range 120 dB (True WDR) ### Storage microSD The camera supports microSD up to 256 GB. The memory card should be Class V30 or faster. The card is not included in the complete set. |
| ### Audio | Built-in microphone With the corresponding settings, the camera can record audio at the scene. Audio codec G711-ULAW, G711-ALAW, G726, G722 |
| ### Intelligent video analytics | Motion detection The camera detects motion using software image frame analysis. AI object detection People Pets Vehicles |
| ### Network | ### Port 1 × RJ-45 self-adaptive Ethernet (10/100 Mbps) ### Protocol HTTP, HTTPS, TCP/IP, IPv4, RTSP, UDP, DHCP, DNS ### ONVIF® conformant with ONVIF® Profile G and S The device implements ONVIF® H.265 (HEVC) encoding, transmission of motion detection and AI recognition events, control over Wide Dynamic Range (WDR), and other image settings — not ONVIF® profile or add-on conformant. |
| ### Key features | ### Video protocol JetSparrow Fast and encrypted peer-to-peer video protocol developed by Ajax Systems. ### Visual verification of Ajax alarms The system allows you to configure a scenario according to which Ajax detectors trigger selected cameras to start recording. The recorded event is immediately delivered to the mobile or desktop monitoring app. ### Privacy Advanced privacy controls ensure a high level of personal data protection. All access rights — including those for mobile app users, installers, and monitoring company operators via Ajax PRO Desktop — are fully configurable by the device owner. This guarantees that only authorized parties can access video streams, event history, or device settings, in accordance with the owner’s preferences. |
| ### Sabotage protection | ### mTLS Encrypted connection for data protection. ### Authentication via Ajax Cloud ### Secure Boot Protection against firmware spoofing. ### OTA signature verification Secure automatic firmware update. |
| ### Power supply | ### PoE standard 802.3at/af ### PoE power consumption - maximum: up to 7.3 W - average: up to 6.0 W ### Power supply unit 12 V⎓ ± 20%, 1 А ### 12 V DC power consumption - maximum: up to 6.3 W - average: up to 5.1 W |
| ### Enclosure | Dimensions 198 × 76 × 76 mm 7.78″ × 2.99″ × 2.99″ Weight 550 g 1.21 lb Operating temperature range from −30 °C to +60 °C from −22 °F to 140 °F Operating humidity up to 90% Protection class IP65, IK08 |
| ### Color | Black White |
| ### Complete set | BulletCam HL (5 Mp/2.8 mm) Installation template Installation kit Quick start guide |
| Warranty | 24 months |