# BulletCam HL (8 Mp/2.8 mm)

Wired AI-powered security IP camera with a 110° viewing angle, hybrid illumination, TrueWDR, microphone, and PoE/12 V. For outdoor and indoor use.

| ### Camera | Image sensor 8 MP CMOS, 1/2.7" type Lens focal length 2.8 mm Maximum aperture f/1.6 Lens iris type fixed Lens focal type fixed Field of view - horizontal: 100–110° - vertical: 50–60° - diagonal: 125–135° DORI distance - Detection: 90 m - Observation: 36 m - Recognition: 18 m - Identification: 9 m Maximum illumination distance - white LED: up to 50 m - IR LED: up to 50 m Intelligent lighting control automatically adjusts the intensity of IR and white LEDs in real time to prevent overexposure. This ensures clear visibility of objects at varying distances — whether close to or far from the camera — in low-light conditions. Minimum illumination color: 0.005 lx B/W: 0 lx Signal-to-noise ratio (SNR) more than 52 dB |
| --- | --- |
| ### Video | Resolution - 8 MP (3840 × 2160) - 6 MP (3072 × 1728) - 5 MP (2592 × 1944) - 5 MP (2944 × 1656) - 5 MP (2880 × 1620) - 4 MP (2560 × 1440) - 3 MP (2304 × 1296) - 2 MP (1920 × 1080) - 0.6 MP (1024 × 576) Frame rate up to 20 fps Image rotation 0°, 180° Video codec H.265, H.264 Dynamic range 120 dB (True WDR) Storage microSD The camera supports microSD up to 256 GB. The memory card should be Class V30 or faster. The card is not included in the complete set. |
| ### Audio | Built-in microphone With the corresponding settings, the camera can record audio at the scene. Audio codec G711-ULAW, G711-ALAW, G726, G722 |
| ### Intelligent video analytics | Motion detection The camera detects motion using software image frame analysis. AI object detection People Pets Vehicles |
| ### Network | ### Port 1 × RJ-45 self-adaptive Ethernet (10/100 Mbps) ### Protocol HTTP, HTTPS, TCP/IP, IPv4, RTSP, UDP, DHCP, DNS ### ONVIF® conformant with ONVIF® Profile G and S The device implements ONVIF® H.265 (HEVC) encoding, transmission of motion detection and AI recognition events, control over Wide Dynamic Range (WDR), and other image settings — not ONVIF® profile or add-on conformant. |
| ### Key features | ### Video protocol JetSparrow Fast and encrypted peer-to-peer video protocol developed by Ajax Systems. ### Visual verification of Ajax alarms The system allows you to configure a scenario according to which Ajax detectors trigger selected cameras to start recording. The recorded event is immediately delivered to the mobile or desktop monitoring app. ### Privacy Advanced privacy controls ensure a high level of personal data protection. All access rights — including those for mobile app users, installers, and monitoring company operators via Ajax PRO Desktop — are fully configurable by the device owner. This guarantees that only authorized parties can access video streams, event history, or device settings, in accordance with the owner’s preferences. |
| ### Sabotage protection | ### mTLS Encrypted connection for data protection. ### Authentication via Ajax Cloud ### Secure Boot Protection against firmware spoofing. ### OTA signature verification Secure automatic firmware update. |
| ### Power supply | ### PoE standard 802.3at/af ### PoE power consumption - maximum: up to 8.1 W - average: up to 7.0 W ### Power supply unit 12 V⎓ ± 20%, 1 А ### 12 V DC power consumption - maximum: up to 7.0 W - average: up to 6.0 W |
| ### Enclosure | Dimensions 198 × 76 × 76 mm 7.78″ × 2.99″ × 2.99″ Weight 550 g 1.21 lb Operating temperature range from −30 °C to +60 °C from −22 °F to 140 °F Operating humidity up to 90% Protection class IP65, IK08 |
| ### Color | Black White |
| ### Complete set | BulletCam HL (8 Mp/2.8 mm) Installation template Installation kit Quick start guide |
| Warranty | 24 months |